Map description and analysis

- Introduction

The Tessta river is known as the right bank tributary of Brahmaputra and the basin covers an area approx. 8205 sq.km of Sikkim-Darjeeling Himalayas. The Teesta river originates from Pauhunri Massif and flows southwards through gorges and rapids of Sikkim Himalays. Teesta basin lies in between Longitude 80°E–88.88°E and Latitude 28.09°N–26.84°N

- Methodology and Datasets

  - Datasets: The Teesta basin is prepared using 4 sets of Cartosat-1 Digital Elevation Models (CartoDEM) obtained from Bhuvan portal(https://bhuvan.nrsc.gov.in)

  - Methodology: The basin visualization map is prepared using 3 open-source softwares-Qgis,   WhiteBoxTools and Blender3D

WhiteBoxTools Steps for Basin Delineation

Mosaic DEM

Reproject

Clip to Boundary

Rescale

Translate- UInt16

Fill Dem

Flow pointer

Dh and Rho8 flow accumulation

Stream generation

Watershed delineation

Add plane

Image Texture using DEM

Add nodes for Displacement and effects

Subdivide into meshes

Render

Qgis Steps for shaded relief creation

Blender Steps for shaded relief creation using cycles render engine creation

- Application and Use of Map

A basin map depicts an area of land where run-off water flows to form a waterway. Basins influence the biodiversity and ecology of a stream and river system. So understanding the bounds and order of the streams is really important to control damaging runoff and to implement a proper watershed management plans.

A basin map with a shaded relief can combine an appealing aesthetics with collective information on the terrain makes the map rich with content and will be readily accessible to a map viewer with less training.